- Detection Performance

- Detection MethodPassive Infrared;Microwave

- Detection Range15m

- Detection Angle90° @ 180° adjustable

- Detection Zones92

- Detectable Speed0.3～2m/s

- SensitivityAuto;Low

- White Light Filter10000lux

- Microwave Frequency24GHz(24.15～24.25GHz)

- Pet Immunity40Kg

- Digital Temperature CompensationSupport

- Anti-maskingSupport

- Adaptable Coverage

Support

Pattern mask included

- Feature

- Digital ProcessingTri-Signal Logic

- Sealed OpticsSupport

- Tamper ProtectionFront;Rear

- Anti-Sway AnalyticsSupport

- Interface

- Alarm OutputNormally Closed

- LED IndicatorGreen(Top PIR), Red(Microwave), Orange(Bottom PIR)Green+Orange(Mask), Blue(Alarm)

- Transmission

- Transmission MethodWired

- Electrical Characteristics

- Power Consumption

30mA （Standard）

220mA Max （With lighting on）

- Power Supply9 to 16 VDC

- Typical Voltage12 VDC

- General

- Operation Temperature-25 °C to 60 °C (-13 °F to 140 °F)

- Storage Temperature-25 °C to 60 °C (-13 °F to 140 °F)

- Operation Humidity10% to 90%

- IP RateIP65

- Dimension(WxHxD)75mm × 202mm × 73mm

- Weight350g

- Mounting Height0.8 to 1.2m

- Mounting MethodWall

- Application ScenarioOutdoor

Wired Triple Signal Detector

- · 90°@180° adjustable coverage

- · IP65 weather resistance for outdoor application

- · Active IR anti-masking

- · Automatic sensitivity

- · Selectable EOL resistors for alarm and tamper-proof wiring